SIMATS ENGINEERING

ASSESSMENT TOOL 2

COURSE NAME:	CSA07 COURSE CODE: Computer Networks

# COURSE OUTCOME COVERED

CO1: Apply the functions of OSI layer in enabling reliable data transmission across networks.

(BL2, BL3)

Assessment Tool Weightage: 30%

Annexure A

# CONCEPT MAPPING

- A partially completed concept map is given below:

Application → Presentation → ______ → Transport → Network → ______ → Physical.

Analyze the missing layers and explain how their functions are essential for reliable

communication.

Missing Layers:

Session Layer

Establishes, manages, and terminates communication sessions.

Provides synchronization between devices.

Data Link Layer

Performs framing, MAC addressing, and error detection.

Ensures reliable node-to-node data transfer.

Importance for Reliable Communication:

Session layer maintains communication between applications.

Data Link layer detects transmission errors and delivers frames correctly.

Together they improve reliability and efficiency.

- Construct a concept map linking data units (Segment, Packet, Frame, Bits) with their respective OSI layers and analyze how transformation of data units supports transmission.

Analysis:

Segment: Transport layer divides data.

Packet: Network layer adds IP address for routing.

Frame: Data Link layer adds MAC address and error detection.

Bits: Physical layer converts data into electrical/optical/radio signals.

This transformation allows efficient and reliable transmission over networks.

3.Given the concept map:

Data Link→ MAC Address→ Local Delivery

Network → IP Address → Global Delivery

Explain how these relationships ensure complete data delivery across networks.

Explanation:

Data Link layer uses MAC addresses to deliver data within the same local network. Network layer uses IP addresses to deliver packets between different networks. Together they ensure complete end-to-end communication.

4.A concept map shows:

Error Control

→DataLinkLayer→ErrorDetection

→ Transport Layer → Error Correction

Analyze how the interaction between these layers improves reliability in data transmission.

Analysis:

Data Link layer detects transmission errors using CRC or checksum.

Transport layer (TCP) corrects errors through acknowledgements and retransmissions. Their combined operation ensures accurate and reliable data delivery.

5.Develop a concept map for web browsing using OSI layers and analyze how failure in

one layer affects the entire communication process.

Analysis:

If any OSI layer fails, communication is interrupted.

Example:

Physical layer failure → No connection.

Network layer failure → Packets cannot reach destination.

Transport layer failure → Connection times out.

Application layer failure → Web page cannot load.

6.A network issue is represented in the concept map:

Physical ✔ → Data Link ✔ → Network ✖ → Transport ✖ → Application ✖

Analyze the problem and explain how the concept map helps in troubleshooting.

Analysis:

Physical and Data Link layers are working correctly.

The problem starts at the Network layer, likely due to:

Incorrect IP address

Routing failure

Gateway problem

Since the Network layer fails, Transport and Application layers also fail.

Troubleshooting Steps:

1. Check IP configuration.

2. Verify default gateway.

3. Test using ping and traceroute.

4. Check router configuration.

5. Restart network devices if required.

# RUBRICS FOR CALCULATION

| Criteria | Weightage (%) | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Concept Identification | 20% | Accurately identifies all relevant OSI layers, concepts, and relationships | Identifies most concepts with minor omissions | Identifies limited concepts; some inaccuracies | Unable to identify key concepts correctly |
| Concept Mapping Structure | 25% | Constructs a clear, logical, and well- organized concept map with proper hierarchy and connections | Concept map is mostly clear with minor structural issues | Concept map lacks clarity, organization, or proper flow | No clear structure or incorrect mapping |
| Linking & Relationships | 20% | Clearly establishes correct relationships between layers, functions, and processes | Relationships are mostly correct with minor errors | Limited or partially correct relationships | Incorrect or missing relationships |
| Application & Analysis | 20% | Effectively applies concepts to explain processes (e.g., encapsulation, error control) with strong reasoning | The application is correct but lacks depth in analysis | Basic application with weak explanation | Unable to apply concepts correctly |
| Explanation & Clarity | 15% | Provides a clear, concise, and wellstructured explanation supporting the concept map | Explanation is understandable with minor gaps | Explanation is unclear or incomplete | No clear or meaningful explanation |