SIMATS ENGINEERING

ASSESSMENT TOOL 2

# Case study

COURSE NAME: Computer Networks	COURSE CODE:	CSA07

## COURSE OUTCOME COVERED

CO3: Analyze the performance of core network protocols such as TCP, UDP, HTTP, and DNS under varying conditions

Assessment Tool Weightage: 40%

# Case study

## QUESTIONS:4	                   Total Marks:40

1.A multinational IT company uses a cloud-based video conferencing platform for daily client meetings. During peak business hours, employees experience frequent voice distortion, frozen video frames, and delayed communication. Network monitoring reports indicate an 8% packet loss, 180 ms jitter, and fluctuating bandwidth utilization. The IT administrator suspects that the issue is related to the transport protocol or application-level buffering strategy.

Questions

- Analyze whether the problem is primarily caused by the transport protocol (TCP/UDP) or the application layer.

- Justify your answer based on the communication requirements of real-time multimedia applications.

- Recommend suitable protocol-level or application-level improvements to enhance conferencing quality.

2.A multinational software company hosts its web services across multiple continents. Employees frequently report long delays before the application homepage loads. Network analysis shows that the average DNS lookup time is approximately 800 ms, significantly affecting user experience. The organization plans to redesign its DNS infrastructure while ensuring continuous service availability during server failures.

Questions

- Analyze the reasons behind the high DNS resolution time.

- Propose a suitable DNS architecture using techniques such as Anycast DNS, DNS pre-fetching, and TTL optimization to reduce the lookup time below 50 ms.

- Evaluate the advantages and limitations of your proposed architecture in terms of performance, scalability, and availability.

3.A cloud service provider delivers applications to millions of users worldwide. During promotional events, users experience intermittent delays in accessing cloud-hosted services. Investigation reveals that DNS queries are taking nearly 800 ms because all requests are directed to a single primary DNS server. The company wants a highly available DNS solution capable of handling global traffic efficiently.

Questions

- Examine the impact of centralized DNS architecture on application performance.

- Design a distributed DNS solution that minimizes lookup latency while maintaining high availability.

- Analyze how TTL values, Anycast routing, and DNS caching influence system performance and fault tolerance.

4.A financial institution operates an electronic stock trading platform where thousands of buy and sell orders are submitted every second. During market opening hours, the average order acknowledgment latency increases from 1 ms to 40 ms, resulting in delayed trade confirmations and customer dissatisfaction. Network engineers observe increased TCP retransmissions, longer connection establishment times, and excessive buffering.

Questions

- Analyze three possible TCP-related factors responsible for the latency increase, considering flow control, Nagle's algorithm, and SYN queue limitations.

- Explain how each factor affects transaction processing in high-frequency systems

- Recommend appropriate TCP configuration changes to reduce latency and improve transaction reliability.

- Cloud-Based Video Conferencing Performance Issues

1. Analyze the Problem

The issue is mainly caused by network conditions affecting UDP and the application's buffering.

- UDP does not retransmit lost packets, so 8% packet loss causes voice distortion and frozen video.

- High jitter (180 ms) causes packets to arrive at different times, leading to delays and interruptions.

- If the jitter buffer is too small, the application cannot smooth out these delays.

2. Justification

Real-time video conferencing requires:

- Low delay

- Smooth audio and video

- Continuous communication

UDP is preferred because it sends data quickly without waiting for lost packets to be retransmitted. This keeps conversations real-time, even if a few packets are lost. Using TCP would increase delays because it retransmits lost packets.

3. Recommendations

- Use RTP over UDP for audio and video.

- Increase or use an adaptive jitter buffer to handle delay variations.

- Apply QoS (Quality of Service) to give voice and video traffic higher priority.

- Increase network bandwidth and reduce congestion.

- Use Forward Error Correction (FEC) to recover some lost packets without retransmission.

Conclusion:
The conferencing problems are mainly due to UDP packet loss and high jitter, along with application-level buffering. Improving network quality and using adaptive buffering will provide smoother voice and video communication.

2. DNS Infrastructure Redesign (Simple Answer)

1. Analyze the reasons behind the high DNS resolution time.

The DNS lookup time is 800 ms, which is very high. Possible reasons are:

- A single DNS server is handling all requests.

- Users are located far from the DNS server.

- DNS caching is not enabled.

- Low TTL settings cause frequent DNS lookups.

- Network congestion during peak hours.

- Too many users accessing the DNS server at the same time.

2. Propose a suitable DNS architecture.

Recommended Solution:

- Use Anycast DNS with multiple DNS servers in different countries.

- Enable DNS pre-fetching to load DNS records before users access the website.

- Set an optimized TTL (e.g., 300–600 seconds) so DNS records remain cached.

- Use backup (secondary) DNS servers for failover.

- Enable DNS caching to reduce repeated lookups.

Simple Architecture

Users

│

DNS Cache

│

Anycast DNS

┌──────┼──────┐

│      │      │

Asia  Europe  America

DNS     DNS     DNS

│

Web Server

This helps reduce DNS lookup time from 800 ms to below 50 ms.

3. Advantages and Limitations

| Advantages | Limitations |
| --- | --- |
| Faster DNS lookup | Higher setup cost |
| Better website performance | More complex management |
| High availability | Requires multiple DNS servers |
| Automatic failover | TTL needs proper configuration |
| Easy to support more users | Maintenance is required |

Conclusion

Using Anycast DNS, DNS caching, DNS pre-fetching, and optimized TTL values reduces DNS lookup time, improves website loading speed, and provides high availability even if one DNS server fails.

3. Cloud Service Provider DNS

4. Impact of Centralized DNS Architecture

- All DNS requests go to one server, causing high traffic.

- DNS lookup time increases (about 800 ms).

- Applications load slowly.

- If the DNS server fails, users cannot access the application.

- Poor user experience during peak traffic.

5. Distributed DNS Solution

- Deploy multiple DNS servers in different locations.

- Use Anycast DNS to send users to the nearest DNS server.

- Enable DNS caching to reduce repeated lookups.

- Set an optimized TTL (e.g., 300–600 seconds) for faster responses.

- Configure automatic failover so another DNS server works if one fails.

6. Effect of TTL, Anycast, and DNS Caching

| Feature | Effect |
| --- | --- |
| TTL (Time To Live) | Reduces repeated DNS queries and improves lookup speed. |
| Anycast Routing | Directs users to the nearest DNS server, reducing latency and balancing traffic. |
| DNS Caching | Stores DNS records locally, making future lookups faster and reducing server load. |

Conclusion

Using distributed DNS, Anycast routing, TTL optimization, and DNS caching reduces DNS lookup time, improves performance, and provides high availability even during heavy traffic.

1. Three TCP-Related Factors Causing Latency

a) Flow Control

- A small TCP receive window slows down data transfer.

- The sender must wait before sending more data, increasing latency.

b) Nagle's Algorithm

- Combines small packets before sending them.

- This causes a delay in sending order acknowledgments.

c) SYN Queue Limitations

- During heavy traffic, the SYN queue becomes full.

- New TCP connections take longer to establish.

2. Effect on High-Frequency Trading

| TCP Factor | Effect on Transactions |
| --- | --- |
| Flow Control | Slows order transmission and acknowledgments. |
| Nagle's Algorithm | Delays sending small trading messages. |
| SYN Queue Limitation | Increases connection setup time and delays new orders. |

3. TCP Configuration Changes

- Disable Nagle's Algorithm (TCP_NODELAY) to send packets immediately.

- Increase TCP send/receive buffer sizes for better data flow.

- Increase the SYN backlog (queue size) to handle more connection requests.

- Use persistent (keep-alive) TCP connections instead of creating new connections for every order.

- Optimize TCP retransmission timeout (RTO) to recover from packet loss faster.

Conclusion

By disabling Nagle's Algorithm, increasing buffer and SYN queue sizes, and using persistent TCP connections, the trading platform can reduce latency and improve transaction reliability during peak trading hours.

RUBRICS FOR CALCULATION

| Criteria | Weightag e | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understanding of Case | 25 | Thorough understanding; clearly identifies all key issues | Good understanding with most issues identified | Basic understandi ng; some issues missed | Poor understanding; problem not identified correctly |
| Application of Concepts | 25 | Effectively applies relevant concepts to analyze the case deeply | Applies appropriate concepts with minor gaps | Limited application of concepts | Fails to apply relevant concepts |
| Analysis | 20 | In-depth analysis with strong reasoning and insights | Good analysis with logical reasoning | Basic analysis with limited depth | Weak or no analysis; lacks reasoning |
| Solution Design | 20 | Innovative, feasible solutions with strong justification | Logical solutions with adequate justification | Basic solutions with weak justification | Incorrect or no solution provided |
| Clarity | 10 | Clear, wellstructured, and easy to understand | Organized and understandable presentation | Limited clarity and structure | Poor clarity; difficult to understand |