SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO5:

Design network topologies star, mesh, bus, and hybrid using networking devices, transmission media, and cabling standards  (BL2,BL5)

Assessment Tool Weightage: 40%

QUESTIONS: 3	Total Marks:30

Annexure A

Constraint-Based Problem Solving

- A  multinational company is opening a new branch office in a remote rural location where high-speed wired internet is unavailable. The office must support cloud-based applications, video conferencing, and secure communication with the headquarters. The company has a limited budget for connectivity but requires uninterrupted business operations. As a network engineer, recommend a suitable internet connection, networking devices, and security mechanisms. Justify your recommendations by considering

bandwidth limitations, reliability, cost, and future scalability

- A university plans to provide campus-wide wireless internet access for more than 3,000 students across classrooms, laboratories, libraries, and hostels. The network must support a large number of simultaneous users while minimizing signal interference and ensuring secure access. Due to budget constraints, only a limited number of wireless access points can be deployed. Analyze the requirements and propose an efficient wireless network design, including access point placement, frequency band selection, and authentication methods. Explain how your solution balances coverage, performance, security, and cost.

- A financial institution is upgrading its internal computer network to improve cybersecurity while allowing employees to access sensitive customer information and online banking services. The organization must comply with strict security policies without significantly affecting network performance or increasing operational costs. Design a secure network architecture by selecting appropriate network segmentation techniques, firewall deployment, authentication methods, and intrusion detection mechanisms. Justify your design by considering the constraints of security, performance, compliance, and budget.

1. Rural Branch Office – Recommended Network Design

Recommended Internet Connection

Primary: 4G/5G fixed wireless broadband
Backup: Satellite internet or second mobile-network connection.

This avoids the need for wired broadband infrastructure and provides reasonable bandwidth for cloud applications and video conferencing.

Networking Devices

- 4G/5G router – Internet connectivity

- Business-grade Wi-Fi 6 access point – Wireless users

- Managed Gigabit switch – Connect PCs, printers, servers, etc.

- Firewall/UTM – Network security

- VPN router/client – Secure connection to headquarters

- UPS – Keeps networking equipment running during short power failures

Security Mechanisms

- Site-to-site IPsec VPN with headquarters

- Firewall with intrusion prevention

- WPA3-Enterprise for Wi-Fi

- Multi-factor authentication (MFA)

- VLAN-based separation of employees, guests, and IoT devices

- Regular firmware updates and monitoring

Justification

| Requirement | Solution |
| --- | --- |
| Bandwidth | 4G/5G provides sufficient bandwidth for cloud and video applications |
| Reliability | Secondary connection provides failover |
| Cost | Cheaper than deploying dedicated wired infrastructure |
| Security | Firewall + VPN + WPA3 protects business data |
| Scalability | Wi-Fi 6, managed switches and VLANs allow future expansion |

Recommendation: Use 5G/4G as primary + satellite/second ISP as backup, with automatic WAN failover.

2. University Campus Wireless Network – 3,000+ Students

Network Design

A centrally managed Wi-Fi 6/Wi-Fi 6E network should be used. Because the number of APs is limited, AP placement should focus on high-density areas rather than simply maximizing coverage.

Access Point Placement

- Classrooms: APs positioned centrally or in corridors serving multiple rooms

- Laboratories: Dedicated APs because of high user density

- Library: Multiple carefully planned APs for simultaneous users

- Hostels: APs distributed floor-wise

- Avoid placing APs close to metal objects and thick concrete barriers

- Perform a wireless site survey before installation

Frequency Selection

- 2.4 GHz: Use mainly for long-range and low-bandwidth devices.

- 5 GHz: Primary band for students because it provides higher speed and more channels.

- 6 GHz: Use where Wi-Fi 6E equipment is available and budget permits.

For 2.4 GHz, use non-overlapping 1, 6, and 11 channels. In dense areas, reduce AP transmit power and carefully reuse channels to minimize co-channel interference.

Authentication and Security

- WPA3-Enterprise

- 802.1X authentication

- RADIUS server

- University ID/password or certificate-based authentication

- Separate guest network

- VLANs for students, staff, laboratories, and guests

- Firewall policies between VLANs

Balancing Requirements

| Requirement | Approach |
| --- | --- |
| Coverage | Strategic AP placement and site survey |
| Performance | Prefer 5 GHz/6 GHz and control channel reuse |
| Interference | Channel planning and reduced transmit power |
| Security | WPA3-Enterprise + 802.1X + RADIUS |
| Cost | Deploy APs mainly in high-density areas |
| Scalability | Central WLAN controller/cloud management |

Recommendation: Use Wi-Fi 6 with 5 GHz as the main student-access band, supported by 2.4 GHz for legacy/low-bandwidth devices and 6 GHz where practical.

3. Financial Institution – Secure Internal Network

Recommended Architecture

Use a segmented, defense-in-depth architecture:

Internet → Edge Router → Next-Generation Firewall → DMZ → Core Switch → VLANs → Internal Users/Servers

Network Segmentation

Create separate VLANs/security zones for:

- Employee computers

- Customer-information systems

- Online banking/application servers

- Database servers

- Management systems

- Guest devices

- Security/monitoring systems

Sensitive databases should not be directly accessible from employee or guest networks.

Firewall Deployment

Deploy a Next-Generation Firewall (NGFW) at the Internet edge with:

- Stateful inspection

- Application control

- IPS

- URL/content filtering

- VPN support

- Logging and monitoring

Public-facing services such as web servers should be placed in a DMZ, isolated from internal databases.

Authentication

- MFA for sensitive systems

- 802.1X for network access

- Role-Based Access Control (RBAC)

- Strong password policies

- Privileged Account Management for administrators

- Least-privilege access

Intrusion Detection and Prevention

Use IDS/IPS to monitor suspicious traffic.

- IDS detects and alerts on attacks.

- IPS can automatically block malicious traffic.

- Centralized logging/SIEM can correlate firewall, server, and authentication events.

Justification

| Requirement | Solution |
| --- | --- |
| Security | VLANs + NGFW + IDS/IPS + MFA |
| Performance | High-performance firewall and controlled traffic flows |
| Compliance | Access control, logging, monitoring and audit trails |
| Cost | Use centralized security appliances and existing managed switches |
| Sensitive data | Isolate databases in protected VLANs |
| Online banking | DMZ + application firewall/security controls |
| Scalability | VLANs and centralized security management support expansion |

Recommended architecture: Internet → NGFW → DMZ → Core Layer → Segmented VLANs → Protected Database/Banking Systems, combined with MFA, 802.1X, RBAC, IDS/IPS, and centralized logging.

RUBRICS FOR CALCULATION

| Criteria | Weightage (%) | Exemplary (4) | Proficient (3) | Developing (2) | Beginning (1) |
| --- | --- | --- | --- | --- | --- |
| Understanding of Problem & Constraints | 20% | Clearly identifies all constraints and fully understands the problem requirements | Identifies most constraints with minor gaps in understanding | Identifies some constraints but misses key aspects | Shows little or no understanding of constraints or problem |
| Problem Decomposition | 15% | Breaks problem into logical, wellstructured subproblems effectively | Breaks problem into sub-parts with some logical flow | Attempts decomposition but lacks clarity or structure | No clear decomposition of the problem |
| Application of Constraints in Solution | 20% | Applies all constraints correctly and consistently in solution design | Applies most constraints correctly with minor errors | Applies some constraints but with noticeable errors | Fails to apply constraints appropriately |
| Logical Reasoning & Strategy | 15% | Uses strong, clear, and efficient reasoning strategies | Uses generally sound reasoning with minor gaps | Reasoning is partially correct but inconsistent | Reasoning is unclear or incorrect |
| Solution Accuracy & Feasibility | 15% | Solution is fully correct, feasible, and optimized | Solution is mostly correct and feasible | Solution has errors or is partially feasible | Solution is incorrect or not feasible |
| Creativity & Innovation | 5% | Demonstrates highly innovative and unique approach | Shows some creativity in approach | Limited creativity shown | No creativity; relies on basic or copied ideas |
| Clarity of Presentation | 10% | Solution is clearly presented with excellent organization and explanation | Presentation is clear with minor issues | Presentation lacks clarity or organization | Poorly presented and difficult to understand |