SIMATS ENGINEERING

ASSESSMENT TOOL 1

Experiments

COURSE NAME: Computer Networks	COURSE CODE:	CSA07

# COURSE OUTCOME COVERED

CO3: Analyze the performance of core network protocols such as TCP, UDP, HTTP, and DNS under varying conditions

Assessment Tool Weightage: 40%

SOFTWARE: CISCO PACKET TRACER, WIRESHARK

EXPERIMENTS

# QUESTIONS:4	                Total Marks:40

- Design a simple LAN consisting of two PCs connected through a switch using Cisco Packet Tracer. Configure appropriate IP addresses for both hosts and verify connectivity using the ping command. Simultaneously capture the packet exchange using Wireshark. Identify the Ethernet frame fields such as Source MAC Address, Destination MAC Address, EtherType, and Frame Check Sequence (FCS). Explain how the Data Link layer uses MAC addressing to ensure successful frame delivery within the local network.

- Create a network topology consisting of two LANs connected through a router in Cisco Packet Tracer. Configure IPv4 addressing, subnet masks, and default gateways for all devices, and verify end-to-end communication using the ping command. Capture the ICMP packets using Wireshark and analyze the IPv4 header fields, including Source IP Address, Destination IP Address, Time-ToLive (TTL), Protocol Number, and Header Length. Explain the role of the Network layer in forwarding packets between different networks.

- Construct a local area network with multiple hosts connected through a switch in Cisco Packet Tracer. Clear the ARP cache on one of the hosts and initiate communication with another host using the ping command. Capture the communication in Wireshark and identify the ARP Request and ARP Reply packets. Analyze the sender and target MAC/IP addresses and explain how the Address Resolution Protocol resolves logical IP addresses into physical MAC addresses before data transmission.

- Develop a simple network consisting of two PCs connected through a router in Cisco Packet Tracer. Configure the required IP addresses and test connectivity using the ping command. Capture the ICMP traffic using Wireshark and identify the Echo Request and Echo Reply messages. Examine important ICMP fields such as Type, Code, Identifier, and Sequence Number, and calculate the Round Trip Time (RTT). Discuss how ICMP assists in network troubleshooting and connectivity verification.

- Create a client-server network in Cisco Packet Tracer by configuring a web server and a client PC. Enable the HTTP service on the server and access the hosted webpage from the client using a web browser. Capture the communication in Wireshark and identify the TCP three-way handshake, followed by the HTTP GET request and HTTP response messages. Analyze the application-layer communication and explain how TCP provides reliable delivery for HTTP traffic through connection establishment, acknowledgments, and ordered data transfer.

Experiment 1: LAN Communication and Ethernet Frame Analysis

Aim

To design a simple LAN consisting of two PCs connected through a switch using Cisco Packet Tracer, configure IP addresses, verify connectivity using the ping command, capture packets using Wireshark, and analyze Ethernet frame fields.

Requirements

- Cisco Packet Tracer

- Wireshark

- 2 PCs

- 1 Switch

- Copper Straight-Through Cables

IP Addressing

| Device | IP Address | Subnet Mask |
| --- | --- | --- |
| PC0 | 192.168.1.10 | 255.255.255.0 |
| PC1 | 192.168.1.20 | 255.255.255.0 |

Procedure

- Open Cisco Packet Tracer.

- Place one switch and two PCs.

- Connect PCs to the switch using straight-through cables.

- Assign IP addresses.

- Open Command Prompt on PC0.

- Execute:

- ping 192.168.1.20

- Start Wireshark capture.

- Repeat the ping.

- Stop capture.

- Observe Ethernet frame fields.

Ethernet Frame Fields

- Destination MAC Address

- Source MAC Address

- EtherType

- Payload

- Frame Check Sequence (FCS)

Observation

| Field | Description |
| --- | --- |
| Source MAC | Sender MAC address |
| Destination MAC | Receiver MAC address |
| EtherType | Indicates IPv4 (0x0800) |
| FCS | Error detection |

Theory

The Data Link layer uses MAC addresses to identify devices within the same LAN. Every Ethernet frame contains the source and destination MAC addresses, allowing switches to forward frames correctly. The FCS detects transmission errors before delivering data.

Result

Successfully created a LAN, verified connectivity using ping, captured packets using Wireshark, and analyzed Ethernet frame fields.

Experiment 2: Router Communication and IPv4 Header Analysis

Aim

To create two LANs connected through a router, configure IPv4 addressing, verify communication, capture ICMP packets, and analyze IPv4 header fields.

Requirements

- Cisco Packet Tracer

- Wireshark

- Router

- Switches

- PCs

IP Addressing

LAN 1

| Device | IP |
| --- | --- |
| PC0 | 192.168.1.10 |
| Router G0/0 | 192.168.1.1 |

LAN 2

| Device | IP |
| --- | --- |
| PC1 | 192.168.2.10 |
| Router G0/1 | 192.168.2.1 |

Subnet Mask:

255.255.255.0

Default Gateway:

PC0:

192.168.1.1

PC1:

192.168.2.1

Procedure

- Connect both LANs through the router.

- Configure router interfaces.

- Assign IP addresses.

- Configure default gateways.

- Ping between PCs.

- Capture packets in Wireshark.

- Analyze IPv4 headers.

IPv4 Header Fields

- Source IP Address

- Destination IP Address

- Version

- Header Length

- TTL

- Protocol

- Header Checksum

Observation

| Field | Example |
| --- | --- |
| Source IP | 192.168.1.10 |
| Destination IP | 192.168.2.10 |
| TTL | 128 or 64 |
| Protocol | 1 (ICMP) |
| Header Length | 20 Bytes |

Theory

The Network layer forwards packets between different networks using IP addresses. Routers examine destination IP addresses and forward packets using routing tables. The TTL field prevents routing loops.

Result

Successfully established communication between two different LANs and analyzed IPv4 header fields.

Experiment 3: ARP Protocol Analysis

Aim

To study ARP Request and ARP Reply messages using Wireshark.

Requirements

- Cisco Packet Tracer

- Wireshark

- Switch

- Multiple PCs

Procedure

- Create LAN.

- Assign IP addresses.

- Clear ARP cache:

- arp -d *

- Start Wireshark.

- Ping another PC.

- Observe ARP packets.

ARP Request

Who has 192.168.1.20?

Tell 192.168.1.10

ARP Reply

192.168.1.20 is at

00:E0:F7:8A:3D:15

ARP Fields

| Field | Description |
| --- | --- |
| Sender MAC | Request sender |
| Sender IP | Sender IP |
| Target MAC | Unknown in request |
| Target IP | Requested IP |

Theory

ARP converts logical IP addresses into physical MAC addresses. Before transmitting an Ethernet frame, the sender broadcasts an ARP Request. The destination replies with its MAC address, enabling frame delivery.

Result

Successfully captured ARP Request and Reply packets and observed address resolution.

Experiment 4: ICMP Packet Analysis

Aim

To analyze ICMP Echo Request and Echo Reply packets and calculate RTT.

Requirements

- Cisco Packet Tracer

- Wireshark

- Router

- Two PCs

Procedure

- Configure router and PCs.

- Verify connectivity.

- Start Wireshark.

- Execute:

- ping <destination IP>

- Observe ICMP packets.

ICMP Fields

| Field | Description |
| --- | --- |
| Type | 8=Request, 0=Reply |
| Code | 0 |
| Identifier | Session ID |
| Sequence Number | Packet Number |

RTT Calculation

RTT = Reply Time − Request Time

Example:

Request:

1.245 ms

Reply:

2.130 ms

RTT = 2.130 − 1.245

= 0.885 ms

Theory

ICMP is used for testing connectivity and diagnosing network issues. The ping utility uses ICMP Echo Request and Echo Reply messages to measure network reachability and delay.

Result

Successfully captured ICMP packets and calculated the Round Trip Time (RTT).

Experiment 5: HTTP Communication using TCP

Aim

To configure a client-server network with a web server, access a webpage, capture HTTP traffic, and analyze TCP and HTTP communication.

Requirements

- Cisco Packet Tracer

- Wireshark

- Web Server

- Client PC

- Switch

IP Addressing

| Device | IP Address |
| --- | --- |
| Server | 192.168.1.100 |
| Client | 192.168.1.10 |

Subnet Mask

255.255.255.0

Procedure

- Create client-server topology.

- Configure server IP.

- Enable HTTP service.

- Configure client IP.

- Open the client web browser.

- Enter:

- http://192.168.1.100

- Start Wireshark.

- Refresh the webpage.

- Observe TCP and HTTP packets.

TCP Three-Way Handshake

Client                Server

SYN  ---------------->

<-------------  SYN-ACK

ACK  ---------------->

HTTP Communication

HTTP GET Request

↓

Server Processing

↓

HTTP Response (200 OK)

Wireshark Observation

TCP

- SYN

- SYN-ACK

- ACK

HTTP

- GET Request

- HTTP Response

- HTML Data

Theory

TCP provides reliable communication through connection establishment, acknowledgments, retransmissions, and ordered delivery. HTTP operates on top of TCP, using the established connection to exchange web requests and responses reliably.

Result

Successfully configured a web server, accessed the hosted webpage from the client, captured the TCP three-way handshake and HTTP messages, and analyzed reliable TCP communication.

# RUBRICS FOR CALCULATION

| Criteria | Weightag e | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understandin g of Concepts | 25 | Demonstrates thorough understanding and effectively integrates multiple concepts/technologi es | Good understanding with proper integration of most concepts | Basic understanding ; limited or partial integration | Poor understanding ; unable to integrate concepts |
| Experimental Design | 30 | Well-planned, systematic implementation with optimal solution | Correct implementati on with minor issues | Basic implementati on with errors or inefficiencies | Incorrect or incomplete implementati on |
| Use of Tools | 20 | Effective and advanced use of tools/technologies with proper configuration | Appropriate use of tools with correct execution | Limited or inconsistent use of tools | Incorrect or minimal use of tools |
| Analysis of Results | 15 | Insightful analysis with accurate interpretation and validation of results | Good analysis with reasonable interpretation | Basic analysis with limited insights | Poor or incorrect analysis of results |
| Documentati on | 10 | Clear, wellstructured documentation with diagrams, code, and results | Organized documentatio n with necessary details | Basic documentatio n with missing details | Poor or incomplete documentatio n |