SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO4:

Design network topologies star, mesh, bus, and hybrid using networking devices, transmission media, and cabling standards  (BL2,BL6)

Assessment Tool Weightage: 40%

Annexure A

Coding challenge

- +Develop a Python program that generates a Star Topology for a given number of hosts connected to a central switch. The program should accept the number of hosts as input and display the topology either in ASCII format or a graphical representation. For each host, print the host label, the name of the central switch, and the corresponding Cat6 UTP cable length connecting the host to the switch. Verify that all hosts are connected through the central switch and explain how the star topology facilitates reliable communication in a local area network.

- Write a Python function named validate_segment(length_m) to

verify whether a given UTP cable segment satisfies the IEEE 802.3 Ethernet standard, which specifies a maximum cable length of 100 meters. The function should return True for valid cable lengths and return False along with an appropriate error message for invalid lengths exceeding the permissible limit. Demonstrate the correctness of the program by executing it with at least three different test cases and interpret the results.

- Develop a Python program to simulate the operation of the Carrier Sense Multiple Access with Collision Detection (CSMA/CD) protocol in a star topology consisting of four hosts connected through a central switch. The program should simulate frame transmission attempts, detect transmission collisions, calculate random back-off intervals, and continue transmission after the waiting period. Record every collision event, transmission attempt, and back-off interval in a log file named csma_log.txt, and analyze how CSMA/CD minimizes data transmission conflicts in Ethernet networks.

- Develop a client-server application using Python's socket library to simulate communication between two hosts connected through a central switch in a star topology. The server should receive frames from the client, forward them appropriately, and maintain a simulated MAC Address Table. After each successful frame transmission, display the updated MAC address table showing the learned MAC addresses and associated ports. Explain how MAC learning and frame forwarding are performed in Ethernet switches.

- Design and implement a Python class named Star Switch that models the basic functionality of an Ethernet switch. The class should include methods such as add_port(host_id) to connect a host, remove_port(host_id) to disconnect a host, and broadcast(frame) to transmit a frame to all connected ports except the source port. The program should maintain a record of active ports and log every broadcast operation, indicating the source host and the destination hosts that successfully receive the frame. Demonstrate the implementation using multiple hosts and explain the significance of broadcasting in Ethernet communication.

1. Star Topology Generator

# Star Topology Generator

def create_star_topology():

n = int(input("Enter the number of hosts: "))

switch = "Central-Switch"

print("\n========== STAR TOPOLOGY ==========")

print("              " + switch)

print("                 |")

for i in range(1, n + 1):

host = f"Host-{i}"

cable_length = 10 + (i * 5)   # Simulated Cat6 cable length

print(f"       {host} --- Cat6 UTP ({cable_length} m)")

print("                 |")

print("\n====================================")

print("All hosts are connected through the central switch.")

print("Maximum allowed Ethernet cable length = 100 m")

create_star_topology()

Sample Output

Enter the number of hosts: 4

========== STAR TOPOLOGY ==========

Central-Switch

|

Host-1 --- Cat6 UTP (15 m)

|

Host-2 --- Cat6 UTP (20 m)

|

Host-3 --- Cat6 UTP (25 m)

|

Host-4 --- Cat6 UTP (30 m)

|

====================================

All hosts are connected through the central switch.

Maximum allowed Ethernet cable length = 100 m

Explanation

- Each host has a separate cable connection to the central switch.

- If one cable fails, the other hosts can continue communicating.

- The switch manages communication between hosts.

- This makes star topology reliable and easy to troubleshoot.

2. UTP Cable Length Validation

# UTP Cable Length Validation

def validate_segment(length_m):

if length_m <= 100 and length_m >= 0:

return True

else:

return False

# Test cases

test_cases = [50, 100, 120]

for length in test_cases:

result = validate_segment(length)

if result:

print(f"{length} m -> VALID: Cable length is within 100 m.")

else:

print(f"{length} m -> INVALID: Cable length exceeds the 100 m limit.")

Output

50 m -> VALID: Cable length is within 100 m.

100 m -> VALID: Cable length is within 100 m.

120 m -> INVALID: Cable length exceeds the 100 m limit.

Interpretation

| Cable Length | Result | Reason |
| --- | --- | --- |
| 50 m | Valid | Below 100 m |
| 100 m | Valid | Equal to maximum limit |
| 120 m | Invalid | Exceeds 100 m |

Conclusion: The function correctly identifies whether the cable segment is within the specified 100 m Ethernet channel limit.

3. CSMA/CD Simulation with Log File

Note: CSMA/CD is historically associated with shared-medium, half-duplex Ethernet. A modern switched full-duplex Ethernet network normally does not use CSMA/CD.

# CSMA/CD Simulation

import random

import time

hosts = ["Host-1", "Host-2", "Host-3", "Host-4"]

log_file = "csma_log.txt"

with open(log_file, "w") as log:

log.write("========== CSMA/CD SIMULATION ==========\n")

for attempt in range(1, 11):

# Select a random host

sender = random.choice(hosts)

log.write(f"\nTransmission Attempt {attempt}\n")

log.write(f"{sender} is sensing the channel...\n")

# Randomly generate collision

collision = random.choice([True, False])

if collision:

log.write(f"COLLISION detected for {sender}\n")

# Random backoff

backoff = random.randint(1, 5)

log.write(

f"{sender} waits for {backoff} back-off time units\n"

)

print(

f"Attempt {attempt}: {sender} -> "

f"Collision -> Back-off = {backoff}"

)

else:

log.write(f"{sender} transmitted successfully\n")

print(

f"Attempt {attempt}: {sender} -> "

f"Transmission successful"

)

log.write("\n========== END OF SIMULATION ==========\n")

print("\nSimulation completed.")

print("Log file created: csma_log.txt")

Example Console Output

Attempt 1: Host-2 -> Transmission successful

Attempt 2: Host-4 -> Collision -> Back-off = 3

Attempt 3: Host-1 -> Transmission successful

Attempt 4: Host-3 -> Collision -> Back-off = 2

Attempt 5: Host-2 -> Transmission successful

...

Simulation completed.

Log file created: csma_log.txt

csma_log.txt

========== CSMA/CD SIMULATION ==========

Transmission Attempt 1

Host-2 is sensing the channel...

Host-2 transmitted successfully

Transmission Attempt 2

Host-4 is sensing the channel...

COLLISION detected for Host-4

Host-4 waits for 3 back-off time units

Transmission Attempt 3

Host-1 is sensing the channel...

Host-1 transmitted successfully

How CSMA/CD works

- Carrier Sense: Host checks whether the channel is free.

- Multiple Access: Multiple hosts share the communication medium.

- Transmission: Host transmits when the channel is free.

- Collision Detection: Host detects a collision during transmission.

- Back-off: Host waits for a random time.

- Retransmission: Host attempts transmission again.

This reduces repeated collisions and improves communication efficiency on shared Ethernet networks.

4. Client-Server Ethernet Switch Simulation

This program uses Python's socket library. The server represents the central switch, while the client represents a host.

Server Program — server.py

import socket

HOST = "127.0.0.1"

PORT = 5000

# Simulated MAC address table

mac_table = {}

server = socket.socket(socket.AF_INET, socket.SOCK_STREAM)

server.bind((HOST, PORT))

server.listen(5)

print("Central Switch Started")

print("Waiting for hosts...")

while True:

connection, address = server.accept()

print("\nHost connected:", address)

data = connection.recv(1024).decode()

if data:

# Message format:

# MAC,PORT,FRAME

parts = data.split(",", 2)

source_mac = parts[0]

source_port = parts[1]

frame = parts[2]

# Learn source MAC

mac_table[source_mac] = source_port

print("\nFrame received:", frame)

print("\nUpdated MAC Address Table")

print("--------------------------")

for mac, port in mac_table.items():

print(f"MAC Address: {mac} | Port: {port}")

connection.send("Frame received and MAC learned".encode())

connection.close()

Client Program — client.py

import socket

HOST = "127.0.0.1"

PORT = 5000

client = socket.socket(socket.AF_INET, socket.SOCK_STREAM)

client.connect((HOST, PORT))

mac_address = input("Enter Host MAC address: ")

port = input("Enter switch port: ")

message = input("Enter frame/message: ")

data = f"{mac_address},{port},{message}"

client.send(data.encode())

response = client.recv(1024).decode()

print("\nSwitch:", response)

client.close()

Example

Run server.py first.

Then run client.py multiple times:

Enter Host MAC address: AA:BB:CC:DD:01

Enter switch port: Port-1

Enter frame/message: Hello from Host-1

Switch: Frame received and MAC learned

Then another host:

Enter Host MAC address: AA:BB:CC:DD:02

Enter switch port: Port-2

Enter frame/message: Hello from Host-2

The switch maintains:

Updated MAC Address Table

--------------------------

MAC Address: AA:BB:CC:DD:01 | Port: Port-1

MAC Address: AA:BB:CC:DD:02 | Port: Port-2

MAC Learning

When a switch receives a frame:

Source MAC → Incoming Port

is stored in the MAC address table.

For example:

AA:BB:CC:DD:01 → Port-1

When a destination MAC is known, the switch can forward the frame only to the corresponding port instead of sending it to every port.

5. StarSwitch Python Class

# StarSwitch Class

class StarSwitch:

def __init__(self):

self.active_ports = []

self.log = []

# Add a host

def add_port(self, host_id):

if host_id not in self.active_ports:

self.active_ports.append(host_id)

print(f"{host_id} connected to switch.")

# Remove a host

def remove_port(self, host_id):

if host_id in self.active_ports:

self.active_ports.remove(host_id)

print(f"{host_id} disconnected from switch.")

# Broadcast frame

def broadcast(self, frame, source_host):

destinations = []

for host in self.active_ports:

if host != source_host:

destinations.append(host)

log_message = (

f"Source: {source_host} | "

f"Frame: {frame} | "

f"Destinations: {destinations}"

)

self.log.append(log_message)

print("\nBroadcast Operation")

print("-------------------")

print("Source Host:", source_host)

print("Frame:", frame)

print("Destination Hosts:", destinations)

# Display active ports

def display_ports(self):

print("\nActive Ports")

print("------------")

for host in self.active_ports:

print(host)

# Create switch

switch = StarSwitch()

# Add hosts

switch.add_port("Host-1")

switch.add_port("Host-2")

switch.add_port("Host-3")

switch.add_port("Host-4")

# Display connected hosts

switch.display_ports()

# Broadcast frames

switch.broadcast("Hello Network", "Host-1")

switch.broadcast("Data Packet", "Host-3")

# Remove a host

switch.remove_port("Host-4")

# Display active ports again

switch.display_ports()

Sample Output

Host-1 connected to switch.

Host-2 connected to switch.

Host-3 connected to switch.

Host-4 connected to switch.

Active Ports

------------

Host-1

Host-2

Host-3

Host-4

Broadcast Operation

-------------------

Source Host: Host-1

Frame: Hello Network

Destination Hosts: ['Host-2', 'Host-3', 'Host-4']

Broadcast Operation

-------------------

Source Host: Host-3

Frame: Data Packet

Destination Hosts: ['Host-1', 'Host-2', 'Host-4']

Host-4 disconnected from switch.

Active Ports

------------

Host-1

Host-2

Host-3

Significance of Broadcasting

Broadcasting allows one Ethernet host to send a frame to all other connected hosts in the broadcast domain.

Example:

Central Switch

/     |     \

/      |      \

Host-1   Host-2   Host-3

|

Broadcast

↓

Host-2 + Host-3

It is useful for network operations such as ARP discovery and other broadcast-based communication. In a switch, broadcast frames are normally forwarded out all relevant ports except the incoming port.

Overall Result

| Experiment | Main Concept | Expected Result |
| --- | --- | --- |
| 1 | Star topology | All hosts connect through central switch |
| 2 | Cable validation | Length ≤ 100 m is accepted |
| 3 | CSMA/CD | Collisions are detected and back-off is applied |
| 4 | Socket communication | MAC addresses are learned by the simulated switch |
| 5 | Ethernet switch class | Frames are broadcast to connected destination ports |

RUBRICS FOR CALCULATION

| Criteria | Weightage | Exemplary (4) | Proficient (3) | Developing (2) | Beginning (1) |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 15% | Fully understands the problem and requirements; no clarification needed | Understands most requirements with minor clarification | Partial understanding; misses some requirements | Poor understanding of the problem |
| Algorithm Design | 20% | Efficient, optimal, and well-structured algorithm | Correct algorithm with minor inefficiencies | Basic approach but not optimal | Incorrect or no clear algorithm |
| Code Quality | 15% | Clean, well organized, readable, and properly formatted code | Mostly clean code with minor issues | Code is somewhat unclear or inconsistent | Poorly written, unreadable code |
| Functionality | 20% | Code works perfectly for all test cases | Works for most test cases | Works for limited cases | Does not run or fails major cases |
| Error Handling | 10% | Handles all edge cases and errors effectively | Handles some edge cases | Limited error handling | No error handling |
| Efficiency | 10% | Highly optimized in time and space complexity | Reasonably efficient | Some inefficiencies | Highly inefficient |
| Documentation & Comments | 5% | Clear and meaningful comments and documentation | Adequate comments | Few or unclear comments | No comments |
| Testing & Debugging | 5% | Thoroughly tested with multiple cases; no bugs | Tested with some cases | Minimal testing | No testing done |